# ESP32 + Ethernet Controlled Servo Actuator for Remote Power Control

## 1. Introduction

This document specifies a low-cost device that remotely controls the physical power button of a computer (e.g., Mac Mini) located in a datacentre or server room. The device uses an ESP32 microcontroller with Wi‑Fi and Ethernet connectivity, exposing a password-protected web interface that commands a compact servo to perform short presses (power toggle) and long presses (forced shutdown/reset).

## 2. Use Case

Designed for administrators who need to power on/off or reset machines without local access. The actuator is mounted to press the rear or underside power button of a Mac Mini (or similar), enabling reliable physical control even when firmware-level options (e.g., Wake-on-LAN) are unavailable.

## 3. Hardware Overview

• MCU: ESP32 module (Wi‑Fi built-in) with external Ethernet via W5500 (SPI) or LAN8720 (RMII).
• Actuator: Micro servo (5 V), sized to fit a compact 3D-printed bracket and lever.
• Power: USB‑C 5 V input; onboard regulation to 3.3 V for digital logic; servo powered from 5 V rail.
• Controls: Reset (EN) and Factory Default buttons; Status LED.
• Indicators: RJ45 link/activity (from magjack).

## 4. Circuit Wiring (Descriptive)

Power enters via USB‑C at 5 V. A 3.3 V regulator supplies the ESP32 and Ethernet interface. The servo uses the 5 V rail with a local bulk capacitor (470–1000 µF) for surge. The web server and configuration logic run on the ESP32. A dedicated PWM GPIO drives the servo’s signal line; grounds are common.

Two Ethernet implementations are provided:
• Variant A (recommended): ESP32 + W5500 (SPI). Fewer pins, simpler layout, robust.
• Variant B: ESP32 + LAN8720 (RMII). Higher throughput interface; requires clock and length-matching.

Buttons: Reset connects to EN (momentary to GND). Factory Default connects to a pulled-up GPIO and, when held on boot (≈10 s), wipes configuration to defaults. Status LED connects to a GPIO via ~330 Ω series resistor.

## 5. Firmware Overview

ESP‑IDF firmware provides: Ethernet (static IP default), Wi‑Fi AP setup/captive portal and STA client, password-protected web UI, REST API, WebSocket status, NVS configuration (user + network), actuator control with timed short/long presses, OTA update, and factory reset logic. Security hardening includes hashed credentials, CSRF on mutating routes, SameSite/HttpOnly cookies, and login rate limiting.

## 6. Mechanical Design

The servo mounts on a compact 3D‑printed bracket that references the case chassis, not the moving button. A lever/horn with a TPU or silicone pad applies ~0.3–1 N over 1–2 mm of travel. Include an adjustable stop (set resting gap to ~0.5–1.0 mm) and a flexure/spring feature for compliance to avoid over‑travel loads on the switch.

## 7. Operation

Via the web UI, the user triggers a Short Press (≈200 ms dwell) or Long Press (≈5 s dwell). The actuator ignores overlapping commands and enforces a brief cooldown. Network, credentials, and timings are configurable from the UI.

## 8. Security

• Password‑protected web UI; credentials stored hashed in NVS.
• CSRF protection on all mutating API routes; SameSite cookies.
• Login rate limiting and optional lockout.
• Optional HTTPS with device certificate or reverse proxy on LAN.

## 9. Summary

This design provides a compact, reliable, network‑controlled servo pusher to physically actuate a Mac Mini’s power button. It is suitable for datacentre deployment and can be adapted mechanically to other equipment.

## 10. Detailed Schematic (Electrical Wiring)

Two supported Ethernet options are shown below. Choose one variant. Pin mappings are provided after each diagram.

### Variant A: ESP32 + W5500 (SPI Ethernet)

Simple SPI wiring; recommended for fast bring‑up. Keep SPI short; decouple VDD with 0.1 µF & 4.7 µF.

| Function | ESP32 GPIO | W5500 Pin | Notes |
| --- | --- | --- | --- |
| SPI MOSI | GPIO23 | MOSI | Series 33–68Ω optional |
| SPI MISO | GPIO19 | MISO | 10k pull‑up optional |
| SPI SCLK | GPIO18 | SCLK | No stubs |
| Chip Select | GPIO5 | SCSn | 10k pull‑up to 3.3V |
| Interrupt | GPIO4 | INTn | Active low; 10k pull‑up |
| Reset | GPIO16 | RSTn | Active low |
| 3.3V | — | VDD | 0.1 µF + 4.7 µF near VDD |
| GND | — | GND | Solid return |
| RJ45 Tx/Rx | — | TX±, RX± | Through magjack |
| Servo PWM | GPIO21 | — | Servo signal |
| Servo +5V | — | — | 470–1000 µF near connector |
| Status LED | GPIO2 | — | 330Ω series |
| Factory Btn | GPIO0 | — | To GND; internal pull‑up |
| Reset Btn | EN | — | Momentary to GND |

### Variant B: ESP32 + LAN8720 (RMII Ethernet PHY)

Use clean 50 MHz reference; length‑match RMII. Follow vendor reference design for straps and magnetics.

| Function | ESP32 GPIO | LAN8720 Pin | Notes |
| --- | --- | --- | --- |
| TXD0 | GPIO19 | TXD0 | Length‑match |
| TXD1 | GPIO22 | TXD1 | Length‑match |
| TX_EN | GPIO21 | TXEN | — |
| RXD0 | GPIO25 | RXD0 | — |
| RXD1 | GPIO26 | RXD1 | — |
| CRS_DV | GPIO27 | CRS_DV | — |
| MDC | GPIO23 | MDC | — |
| MDIO | GPIO18 | MDIO | Pull‑up 1.5–4.7k |
| REFCLK 50 MHz | GPIO0 / ext | XTAL1/CLKIN | Prefer ext 50 MHz |
| 3.3V | — | VDDIO/VDDCR | 0.1 µF + 10 µF |
| GND | — | GND | Solid plane |
| RJ45 MDI | — | TXP/TXN, RXP/RXN | Via magjack; TVS @ jack |
| Servo PWM | GPIO17 | — | Servo signal |
| Status LED | GPIO2 | — | 330Ω series |
| Factory Btn | GPIO5 | — | To GND; internal pull‑up |
| Reset Btn | EN | — | Momentary to GND |

## 11. Protection & Power Notes

• Place a TVS diode at USB‑C 5 V input and an ESD array at RJ45. • Recommended regulator: ≥600 mA 3.3 V LDO/buck; check transient response (Wi‑Fi bursts). • Keep servo on 5 V with its own bulk capacitor; route high‑current returns away from signal grounds. • If servo noise couples into ESP32, add an LC filter (≈2.2 µH + 100 µF) to the servo branch.