# ESP32 + Ethernet Controlled Servo Actuator for Remote Power Control

## 1. Introduction

This document specifies a low-cost device that remotely controls the physical power button of a computer (e.g., Mac Mini) located in a datacentre or server room. The device uses an ESP32 microcontroller with Wi‑Fi and Ethernet connectivity, exposing a password-protected web interface that commands a compact servo to perform short presses (power toggle) and long presses (forced shutdown/reset).

## 2. Use Case

Designed for administrators who need to power on/off or reset machines without local access. The actuator is mounted to press the rear or underside power button of a Mac Mini (or similar), enabling reliable physical control even when firmware-level options (e.g., Wake-on-LAN) are unavailable.

## 3. Hardware Overview

• MCU: ESP32 module (Wi‑Fi built-in) with external Ethernet via W5500 (SPI) or LAN8720 (RMII).
• Actuator: Micro servo (5 V), sized to fit a compact 3D-printed bracket and lever.
• Power: USB‑C 5 V input; onboard regulation to 3.3 V for digital logic; servo powered from 5 V rail.
• Controls: Reset (EN) and Factory Default buttons; Status LED.
• Indicators: RJ45 link/activity (from magjack).

## 4. Circuit Wiring (Descriptive)

Power enters via USB‑C at 5 V. A 3.3 V regulator supplies the ESP32 and Ethernet interface. The servo uses the 5 V rail with a local bulk capacitor (470–1000 µF) for surge. The web server and configuration logic run on the ESP32. A dedicated PWM GPIO drives the servo’s signal line; grounds are common.

Two Ethernet implementations are provided:
• Variant A (recommended): ESP32 + W5500 (SPI). Fewer pins, simpler layout, robust.
• Variant B: ESP32 + LAN8720 (RMII). Higher throughput interface; requires clock and length-matching.

Buttons: Reset connects to EN (momentary to GND). Factory Default connects to a pulled-up GPIO and, when held on boot (≈10 s), wipes configuration to defaults. Status LED connects to a GPIO via ~330 Ω series resistor.

## 5. Firmware Overview

ESP‑IDF firmware provides: Ethernet (static IP default), Wi‑Fi AP setup/captive portal and STA client, password-protected web UI, REST API, WebSocket status, NVS configuration (user + network), actuator control with timed short/long presses, OTA update, and factory reset logic. Security hardening includes hashed credentials, CSRF on mutating routes, SameSite/HttpOnly cookies, and login rate limiting.

## 6. Mechanical Design

The servo mounts on a compact 3D‑printed bracket that references the case chassis, not the moving button. A lever/horn with a TPU or silicone pad applies ~0.3–1 N over 1–2 mm of travel. Include an adjustable stop (set resting gap to ~0.5–1.0 mm) and a flexure/spring feature for compliance to avoid over‑travel loads on the switch.

## 7. Operation

Via the web UI, the user triggers a Short Press (≈200 ms dwell) or Long Press (≈5 s dwell). The actuator ignores overlapping commands and enforces a brief cooldown. Network, credentials, and timings are configurable from the UI.

## 8. Security

• Password‑protected web UI; credentials stored hashed in NVS.
• CSRF protection on all mutating API routes; SameSite cookies.
• Login rate limiting and optional lockout.
• Optional HTTPS with device certificate or reverse proxy on LAN.

## 9. Summary

This design provides a compact, reliable, network‑controlled servo pusher to physically actuate a Mac Mini’s power button. It is suitable for datacentre deployment and can be adapted mechanically to other equipment.

## 10. Detailed Schematic (Electrical Wiring)

Two supported Ethernet options are shown below. Choose one variant. Pin mappings are provided after each diagram.

### Variant A: ESP32 + W5500 (SPI Ethernet)

Simple SPI wiring; recommended for fast bring‑up. Keep SPI short; decouple VDD with 0.1 µF & 4.7 µF.

| Function | ESP32 GPIO | W5500 Pin | Notes |
| --- | --- | --- | --- |
| SPI MOSI | GPIO23 | MOSI | Series 33–68Ω optional |
| SPI MISO | GPIO19 | MISO | 10k pull‑up optional |
| SPI SCLK | GPIO18 | SCLK | No stubs |
| Chip Select | GPIO5 | SCSn | 10k pull‑up to 3.3V |
| Interrupt | GPIO4 | INTn | Active low; 10k pull‑up |
| Reset | GPIO16 | RSTn | Active low |
| 3.3V | — | VDD | 0.1 µF + 4.7 µF near VDD |
| GND | — | GND | Solid return |
| RJ45 Tx/Rx | — | TX±, RX± | Through magjack |
| Servo PWM | GPIO21 | — | Servo signal |
| Servo +5V | — | — | 470–1000 µF near connector |
| Status LED | GPIO2 | — | 330Ω series |
| Factory Btn | GPIO0 | — | To GND; internal pull‑up |
| Reset Btn | EN | — | Momentary to GND |

### Variant B: ESP32 + LAN8720 (RMII Ethernet PHY)

Use clean 50 MHz reference; length‑match RMII. Follow vendor reference design for straps and magnetics.

| Function | ESP32 GPIO | LAN8720 Pin | Notes |
| --- | --- | --- | --- |
| TXD0 | GPIO19 | TXD0 | Length‑match |
| TXD1 | GPIO22 | TXD1 | Length‑match |
| TX_EN | GPIO21 | TXEN | — |
| RXD0 | GPIO25 | RXD0 | — |
| RXD1 | GPIO26 | RXD1 | — |
| CRS_DV | GPIO27 | CRS_DV | — |
| MDC | GPIO23 | MDC | — |
| MDIO | GPIO18 | MDIO | Pull‑up 1.5–4.7k |
| REFCLK 50 MHz | GPIO0 / ext | XTAL1/CLKIN | Prefer ext 50 MHz |
| 3.3V | — | VDDIO/VDDCR | 0.1 µF + 10 µF |
| GND | — | GND | Solid plane |
| RJ45 MDI | — | TXP/TXN, RXP/RXN | Via magjack; TVS @ jack |
| Servo PWM | GPIO17 | — | Servo signal |
| Status LED | GPIO2 | — | 330Ω series |
| Factory Btn | GPIO5 | — | To GND; internal pull‑up |
| Reset Btn | EN | — | Momentary to GND |

## 11. Protection & Power Notes

• Place a TVS diode at USB‑C 5 V input and an ESD array at RJ45. • Recommended regulator: ≥600 mA 3.3 V LDO/buck; check transient response (Wi‑Fi bursts). • Keep servo on 5 V with its own bulk capacitor; route high‑current returns away from signal grounds. • If servo noise couples into ESP32, add an LC filter (≈2.2 µH + 100 µF) to the servo branch.

## Finalized Variant: ESP32 + W5500 (SPI Ethernet)

This section locks the design to the W5500 SPI Ethernet interface. It includes exact GPIO mapping, power tree, protection parts, servo connector specification, and a bring‑up test plan. Use this as the implementation baseline.

### W5500 Pin Mapping (Final)

| Function | ESP32 GPIO | W5500 Pin / Node | Notes |
| --- | --- | --- | --- |
| SPI MOSI | GPIO23 | MOSI | Series 33–68 Ω optional near ESP32 |
| SPI MISO | GPIO19 | MISO | Optional 10 kΩ pull‑up to 3.3 V |
| SPI SCLK | GPIO18 | SCLK | Keep short, no stubs |
| Chip Select | GPIO5 | SCSn | 10 kΩ pull‑up to 3.3 V |
| Interrupt | GPIO4 | INTn | Active‑low; 10 kΩ pull‑up |
| Reset | GPIO16 | RSTn | Active‑low; allow manual reset via MCU |
| 3.3 V | — | VDD | 0.1 µF + 4.7 µF at each VDD pin |
| GND | — | GND | Solid ground plane |
| RJ45 Pairs | — | TX±, RX± | Magjack with integrated magnetics |
| Status LED | GPIO2 | — | 330 Ω to LED anode → GND |
| Factory Button | GPIO0 | — | Momentary to GND, internal pull‑up |
| Reset Button | EN (ESP32) | — | Momentary to GND |
| Servo PWM | GPIO21 | — | To servo signal (5 V tolerant at servo) |
| Servo +5 V | — | — | Dedicated 470–1000 µF bulk cap near header |

### Preliminary BOM (W5500 Variant)

| Item | Part / Example | Qty | Notes | Footprint |
| --- | --- | --- | --- | --- |
| MCU | ESP32‑WROOM‑32E | 1 | Wi‑Fi, 3.3 V logic | Castellated module |
| Ethernet | W5500 | 1 | SPI Ethernet controller | QFN/TQFP |
| RJ45 Magjack | Hanrun HR911105A (or equiv.) | 1 | Integrated magnetics + LEDs | RJ45 TH |
| USB‑C Receptacle | USB Type‑C 16‑pin | 1 | 5 V only, no PD | SMD/TH hybrid |
| CC Resistors | 5.1 kΩ 1% to CC1/CC2 | 2 | Advertise as UFP (sink) | 0603 |
| TVS (USB) | SMAJ5.0A (or SMF5.0A) | 1 | 5 V rail surge clamp | SMA/SMF |
| ESD (RJ45) | Semtech RClamp0512TQ (or Littelfuse SP3012‑06UTG) | 1 | Ethernet ESD protection | QFN/µDFN |
| Regulator 3.3 V | AP2112K‑3.3 (600 mA) or MP1584 buck | 1 | ESP32 + W5500 rail | SOT‑23‑5 / Module |
| Bulk Caps | 100 µF 6.3 V low‑ESR | 2 | 5 V & 3.3 V rails | 1206/1210 |
| Bypass Caps | 0.1 µF X7R | 8–12 | Near VDD pins | 0603 |
| Servo Bulk | 470–1000 µF 6.3 V | 1 | Near servo header | Radial/SMD |
| Status LED | Any 0603 LED | 1 | Green or blue | 0603 |
| LED Resistor | 330 Ω 1% | 1 | LED series | 0603 |
| Factory Button | Tactile, top‑actuated | 1 | GPIO0→GND | SMD/TH |
| Reset Button | Tactile, top‑actuated | 1 | EN→GND | SMD/TH |
| Servo Header | JST‑PH‑3 (2.0 mm) | 1 | GND / +5 V / Signal | TH |
| Mounting | M2.5/M3 standoffs | 4 | Board mounting | — |

### Servo Connector Specification (JST‑PH‑3)

Pinout (board silkscreen from left to right when header key notch faces up):
• Pin 1: GND (black/brown)
• Pin 2: +5 V (red)
• Pin 3: SIGNAL (white/orange) → ESP32 GPIO21 via 100 Ω series (optional)

Recommendations: place the 470–1000 µF bulk capacitor within 10–20 mm of the header; route the signal away from the RJ45 and high di/dt loops. Provide a 3‑pin 2.54 mm adapter on the harness if needed for common servo cables.

### Power Tree & Protection

USB‑C 5 V → TVS (SMAJ5.0A) → main 5 V rail → branches:
• Branch A: 3.3 V regulator → ESP32 + W5500 + logic (decouple with 0.1 µF per VDD and 4.7–10 µF per IC).
• Branch B: Servo 5 V with 470–1000 µF bulk near connector. Consider LC filter (≈2.2 µH + 100 µF) if noise affects logic.
RJ45: place ESD array (RClamp0512TQ or SP3012‑06UTG) at connector pins; short return to chassis/ground.

### Bring‑Up & Test Checklist (W5500)

1) Power‑only test: no servo connected. Verify 5.0 V and 3.3 V rails; ripple under Wi‑Fi burst <100 mVpp.
2) Flash firmware; confirm web UI on Ethernet (default static IP) and Wi‑Fi AP SSID.
3) SPI loop: read W5500 version register (0x0039 expected = 0x04).
4) Link check: RJ45 LEDs and DHCP/static IP; ping from laptop.
5) Servo test: connect servo, verify idle current and motion at 1.0–2.0 ms PWM; tune ‘short’ (200 ms) and ‘long’ (5 s).
6) Factory reset: hold GPIO0 button during boot ≥10 s; confirm defaults restored.
7) EMI sanity: actuate servo repeatedly while streaming pings; check for packet loss; add LC filter if needed.